Version 3.00Beta June 19, 1997

NOTE: This document is also in Beta form right now.

========================================================================

Thanks to Lance Rose for sharing his results in decoding the format of

SA3 startup files.

========================================================================

DISCLAIMER:

As part of it's functionality, g7to creates MapDoc (*.sa3 and/or *.sa4)

files from GPS data.  You can then display the data overlaid on a SA3 or

SA4 map, but it is not real time and should not be used for critical

navigation.

In my opinion, the Lat/Long grid shown in SA3/4 is not accurate enough

for critical navigtion applications (and the license agreement appears

to prohibit using maps from SA3 for navigation).  For instance, in the

Corvallis area of Oregon SA3/4 is in error about 150 feet to the East at

magnification 16.  For example, plotting a known Lat/Long on SA3/4

results in a point on the SA map that is ~150 feet East of it's actual

location.  The error varies from place to place and with the magnitude

of the displayed map.  I've heard rumors that SA3 is up to 1/4 mile off

in some areas, but I've not personally observed an error that large.

There are no guarantees expressed or implied with this g7to and/or it's

documentation.  The package works for me, it may or may not work for

anyone else.  Proper use could corrupt the data stored in your GPS or

on your computer.

The Control point, Proj, Garmin64 and navigate.exe support has not been

fully tested since version 2.05.  I no longer use those formats.

========================================================================

NOTE:  For this program, 'download' is defined as 'from the GPS to the

PC' and 'upload' is 'from the PC to the GPS'.  All references to

the GPS 45 apply to the 12XL.  This program may work with other

Garmin receivers, but it has not been tested by the author.

Versions greater than 2.04 have been tested on the 12XL only.

Program Features:

G7TO will upload/download Track/Waypoint/Route data to/from a

Garmin GPS45.

Data from the GPS45 may be displayed, redirected to a text file,

and/or sent to a Street Atlas 3.0 startup file, otherwise known as

MapDocs.

Versions 2.06a and above include some support for Street Atlas 4.0.

Version 2.08 and above contain some support for the data files

created by WSII, Lowrance's program for communicating with their

GPS's.

Version 3.0 and above allow multiple input and multiple output

files on the command line.

Almanac data from the GPS45 may be downloaded, but not uploaded.

The current position and time can be retrieved from the GPS45.

This function can be set to repeat every 'n' seconds.

The Garmin internal and external voltages can be read.

The Backlight can be turned on and off.

On the Garmin 45 and 12XL units the LCD contrast can be set.

Proximity waypoints:

Proximity waypoints may not be sent to Proj, Garmin64 or .SA3

files.  The distance parameter stored in the text file is in

meters.

If a proximity waypoint is sent to a SA4 file, a circle is drawn.

If a waypoint exists which has the same coordinants as the center

of the circle, it is used as the identifier for the proximity

circle.  If such a waypoint does not exist, one is created with the

name PROXxx where xx is a number representing the xth waypoint

created.

If the waypoint in the center of the circle is moved so that it no

longer resides at the center of the circle and the file is then

downloaded by g7to a pseudo proximity waypoint will be created.

In other words, if a waypoint is tied to a circle and becomes a

proximity waypoint if and only if the waypoint is at the exact

center of the circle.

Note:  Uploading proximity waypoints to the Garmin 12XL first

deletes all proximity waypoints and adds those in the

current upload.  This means that you can have 8 proximity

waypoints stored in the 12XL, send one new proximity

waypoint and end up with only the new one, not 9 in the

12XL.

Waypoints, icons and routes may be read from a Lowrance WSII data

file 'file.low'.  WSII version 1.1 used with a Global Map Sport

version 2.1 has been tested.  Other versions of WSII software and

Global Map Sport GPS units have not been tested.  The .low file has

been tested with the Windows 95 version of WSII only.

========================================================================

Supported Input Formats:

1.  Text, Precision Mapping's Compass waypoint files, ?.wpt,

GarDown7 raw format, Proj format, Garmin64 format,

navigate.exe routes file, routes.clg and Lowrance WSII

files 'fname.low' (waypoints, icons and routes only).  For

input, GarDown7, Proj and Garmin64 are all read as text

files.

If the input file is Garmin64 or Proj format only the Track

and Waypoint records are processed.

2.  SA3 startup files (fname.sa3).  All symbols are output as

Waypoints.  The first 6 characters of an object's name

becomes the waypoint name.  The 7th character is skipped.

Therefore, it is a good idea to make the 7th character a

';' or other character that isn't actually part of the

useful info.  The 8th->Xth character becomes the waypoint

comment.  The waypoint time is taken from the time stamp on

the input file.  Note: the 'XL' series of receivers do not

utilize the time.

X==16 for Text output.

X==19 for PROJ output format.

X==20 for Garmin64 output format.

X==23 for Control Point Record (*80*) format.

3.  SA4 startup files (fname.sa4).  As of version 2.07 the

supported items are lines and areas -- which are sent to

the text file as track data, circles which are sent as

proximity waypoints, the normal waypoints previously

supported in SA3 files.

The SA4 route is supported and routes 0-19 in the Garmin

are supported with up to 30 waypoints in each route.

The Garmin route 0 is also supported as the SA4 route.  The

start and first points are identical as are the finish and

last point.  This results in a route labeled with numbers

only when viewed in SA4.

4.  The GPS 45 via COM port.  Standard IRQ/IO supported.  The

GPS 45 I/O SETUP must be set to GRMN/GRMN and HOST.

Some Garmin units do not transmit the correct date/time

with a record, they send a 0.  For those situations, the

time of a record is taken from the PC system clock at the

time the record is read by the PC.

========================================================================

Supported Output Formats:

1.  SA3 startup files (fname.sa3).  The default object is a

small circle.  The default text location is NE of the

object.  The '.sa3' suffix is not added by the program, it

must be input on the command line.

2.  SA4 startup files (fname.sa4).

Tracks are converted to SA4 lines, proximity waypoints to

circles and routes 0-19 are converted to symbols with a

specific text format.

Provision is made to change the parameters of lines and

circles via '!'  command lines.

3.  Text file.

4.  PROJ.EXE and/or NAD2NAD.EXE format (fname.prj).  See the

documentation for PROJ for further information

5.  Control Point Record (*80*) (fname.x80) for use with the

program UTMS.EXE.  See UTMS documentation for further

information.  !!Output Only!!

6.  Garmin64 format (fname.g64).  See the GARMIN64

documentation for an explanation of this format.

7.  The GPS 45 via COM port, the file name MUST be G45x.  See

below for options on '-i G45x'.

8.  Navigate.exe routes.clg routes file.

9.  WSII from Lowrance files (fname.low).  Waypoints only.

Routes, tracks and icons are not written.

10.  Precision Mapping's Compass addon waypoint files, ?.wpt.

===========================================================================

c:\g7to -hi

Allowable Input files:

SA, text, [Proj, Garmin64 (Waypoint/Track only)], route.clg or GPS45

Use -1 or -2 to set standard COM Port 1-2

To specify the port parameters use:

-3 Port,IOaddr,IRQ (no spaces allowed) where:

Port is 1-4, IOaddr in hex (0xnnn) or decimal, IRQ (3-15) decimal

g7to -3 1,0x3f8,4       defines a standard COM1 port

Use:

-i G45Pn -- Send Time/Position record every n seconds (press 'x' to stop)

-i G45A  -- Send Almanac data

-i G45I  -- Send current Time

-i G45D  -- Send software iD

-i G45R  -- Send Routes

-i G45T  -- Send Tracks (-d to sum distances)

-i G45W  -- Send Waypoints

-i G45X  -- Send proXimity waypoints

-i G45V  -- Send Voltages

-e Do not print symbol attributes to text file

===========================================================================

c:\g7to -ho

-o G45O GPS45 Off; -o G45Cx set contrast x=0 to 15 (tested on 12XL only)

-o G45L1 Light on; -o G45L0 Light off

-----------------------------------------------------------------

-o <fname> - name for output file  (?.sa3, ?.sa4, ?.prj ?.g64 ?.x80)

a - Text alignment:  NW N NE W E SW S SE

b - Object Code.  Values 0-56 see G7TO.DOC

c - ?color ?=T for Text color, O for Object Color

Allowable colors: black,red,blue,green,yellow

f - Do not sort waypoints and do not remove duplicate waypoints

g - map magnitude

k - Line (track) Parameters - C-color, K-thickness, S-style

l - 1st-center map Lat|2nd-center map Long (DDdMM.mmmm)

n - No name and no description

s - ?size ?=T for Text size, O for Object size

Allowable sizes: vs,s,m,l,vl

wx- (x=1,2 or 3) some comments stripped from waypoints. See g7to.doc

r - Draw route lines for SA4 routes 1-19

t - Text output

===========================================================================

c:\g7to -hd

Size     : vs, s, m, l, vl

Alignment: nw, n, ne, w, e, sw, s, se

Color    : black, red, blue, green and yellow

Codes for use with -b:

0 = Agricultural                            35 = One Way Up

1 = Airport        18 = Education           36 = Park

2 = Amusement      19 = Emergency           37 = Parking

3 = Arrow, Down    20 = Fishing             38 = Phone

4 = Arrow, Left    21 = Fuel                39 = Picnic

5 = Arrow, Right   22 = Golf Course         40 = Post Office

6 = Arrow, Up      23 = Government Building 41 = Power

7 = Bank           24 = Historic Site       42 = Religious Star

8 = Beach          25 = Home                43 = Religious Cross

9 = Boat Ramp      26 = Industrial          44 = Restaurant

10 = Campground     27 = Information         45 = Sports

11 = Cemetary       28 = Library             46 = Square

12 = Circle         29 = Lodging             47 = Star

13 = Construction   30 = Mountain            48 = State Capitol

14 = Cross          31 = National capitol    49 = Stop Sign

15 = Delivery Stop  32 = One Way Down        50 = Triangle Down

16 = Diamond        33 = One Way Left        51 = Triangle Up

17 = Doctor/Dentist 34 = One Way Right       52 = Zoo

53 = Bike           54 = Bridge              55 = Church 56 = Police

===========================================================================

Typical usage:

INPUT -- GPS45-wpts, trks, routes, Prox wpts with output .sa3, sa4 and

text.  (a download)

g7to -i G45W -i g45r -i g45t -ig45x -o file.sa3 -o file.sa4 -t >file.txt

will download waypoint, route, track and proximity waypoint data from

the GPS45 and store it in the SA3 startup file file.sa3, file.sa4 and also

write it to the text file file.txt.

g7to -o G45 -i file.sa3 -i file.sa4 -t >waypoint.txt

uploads waypoint data to the GPS45 from the SA3 startup file file.sa3

and also writes it to the text file waypoint.txt.  By default,

waypoint data is sorted and waypoints with the same name are removed.

To disable sorting and duplicate name removal, specify -f on the

command line, i.e.

g7to -f -o G45 -i file.sa3 -i file.sa4 -t >waypoint.txt

If no output file is specified then the output is assumed to be text.

If a COM port is required and none is specified then COM1 is assumed.

g7to -i G45R -o routes.sa4 -t >routes.txt

uploads the routes in the Garmin to a SA4 file and creates an

intermediate text file, routes.txt.

g7to -i G45T -i G45W -i G45R -i G45X -o Garmin.sa4 -t >Garmin.txt

uploads the tracks, routes, waypoints, proximity waypoints in the

Garmin to a SA4 file and creates an intermediate text file,

Garmin.txt

g7to -i waypoint.txt -o waypoint.low -o waypoint.wpt

will read file waypoint.txt and create waypoint.low which will

contain the waypoints only.  The waypoint names in waypoint.txt are

ignored and the waypoint comment (after the date string in the text

file) are transferred as the Lowrance WSII waypoint name.

the -o waypoint.wpt will cause g7to to create the file waypoint.wpt

which can be read into Precision Mapping's Compass addon.

g7to -i waypoint.low

This will read the waypoint and route data from the file waypoint.low

and display it on the screen.  The format is normally 'W xxx..'.  If

a -u is added to the command line the output is the 'L' 'L xxx..'

format.  The 'W' format will allow data from a Lowrance file to be

output to a text file which can be sent to a Garmin unit.  If the

'L' format is used the waypoint icon is appended to the record.

NOTE:  The 'normal' output the name of the waypoint is the first 6

characters of the waypoint name as stored in the .low file.

As Lowrance units allow more than 6 characters, some of the

waypoints in the text file may have the same name.  When

uploaded to the Garmin units the last one uploaded will be ok,

the others will be deleted.  It's a good idea to take a look

at the text file, looking for duplicate waypoint names, before

uploading.  Adding a -u to the command line will output a 'L'

record which has room for 10 characters in the name.  This

format is used only for textfile<->.low file operations.

------------------------------------------------------------------------

INPUT -- text format, OUTPUT SA3 Text is blue, Text size medium

g7to -i waypoint.txt -o waypoint.sa3 -c Tblue -s Tm

------------------------------------------------------------------------

INPUT -- .sa3, OUTPUT -- CPR (*80*) format

g7to -i myfile.sa3 -o myfile.x80

will read the SA3 startup file myfile.sa3 and write its contents to

the file 'myfile.x80 in Control Point Record format.  It will also

output the contents of myfile.sa3 to the screen in normal text format.

g7to -i myfile.sa3 -o myfile.x80 -t

------------------------------------------------------------------------

INPUT -- SA file, output textfile, output an attribute command

line each time an attribute changes.

g7to -i waypoint.sa3 -o waypoint.txt

The 'command line' output will be similar to:

! -b 12 -c oblack -c tblue -s ovs -s tm -a ne

This line signifies that all points in the file following the

command line will be object #12 (circle) the circle will be

black, the text will be blue, the object will be very small and

the text size will be medium.  The text alignment will be North

East of the object.

g7to -i waypoint.sa4 -o waypoint.txt

Line (track) options:

! -k cblue -k s*h -k wvt

This line indicates that lines following this command are blue

(-k cblue), highlighted (-k s*h) and very thin (-k wvt)

*  -k cxx = set line color

*  -k wxx = set line width

*  -k sxx = set line style

* allowable thickness: VT T M K VK

* allowable colors   : black red blue green yellow

* allowable styles   : normal hatched *highlight

Circle options:

! -q ly -q fy -q cblue -q wvt -q ared -q s*h

draw circle with very thin blue line, fill circle with color red

highlighted

*  -q lxx = set circle outer line draw no/yes

*  -q fxx = set circle fill no/yes

*  -q cxx = set circle line color

*  -q wxx = set circle line thickness

*  -q axx = set circle area fill color

*  -q sxx = set circle area fill style

* allowable thickness: VT T M K VK

* allowable colors   : black red blue green yellow

* allowable styles   : normal hatched *highlight

* allowable drawline : no yes

* allowable drawfill : no yes

No object info in text file:

g7to -i waypoint.sa3 -o waypoint.txt -e

This command line will process the .SA3 file but all objects

will be the default object.

------------------------------------------------------------------------

INPUT  Lowrance WSII data file

g7to -i waypoint.low -u

will print the waypoints in the file:

L  ALSEA-BAY  N44 25.8961 W124 04.2498 Thu May 29 15:57:42 1997 ALSEA BAY BRIDGE

------------------------------------------------------------------------

The 'sa output only' options -a,-b,-c,-g,-l,-n,-s, and -wx

-w1- The text records for regular waypoints in the output sa

file will be contain only the waypoint name.  This makes it

easier to see the map surrounding a point, but the user

must rely on the 6 character waypoint name for waypoint

identification.

-w2- Same as -w1 only for route waypoints '(rr:pp)'

-w3- Both route waypoints and regular waypoints will have the

comment stripped leaving only the waypoint name.

NOTE: -n is for sa3 only as tracks in sa4 are actual lines.

-n - Points from a track file do not contain text information,

and the 'waypoint name' normally becomes the position in

the source file.  For instance, if you have 10 points in

your track source file then the points are 'named' 1-10.

-n suppresses this number and only the default symbol, a

small circle, is displayed.  -n also overrides the -w

option and on a waypoint will pass on only the default

symbol.  This works with GarDown7 or text input, not on a

merge.

-k set Line draw parameters (SA4 only):

-k cxx = set line color

-k wxx = set line width

-k sxx = set line style

allowable thickness: VT T M K VK

allowable colors   : black red blue green yellow

allowable styles   : normal hatched *highlight

-a <align> - Text alignment valid parameters are: NW N NE W E SW S SE

-c [T|O]<color> - Object and Text color.  First character is a

'T' for Text color or 'O' for object color.  Valid

colors are black,red,blue,green,yellow.

-s [T|O]<size> - Object and Text size.  First character is a 'T'

for Text size or 'O' for object size.  Valid sizes are

vs,s,m,l,vl (for very small, small, medium, large, very

large).

-b <object#> - See below for valid object codes.

-g <maGnitude> Magnitude of the output .sa3 file

-l <Lat> -l <Long> - The first time the -l parameter appears on

the command line it is assumed to be the desired latitude of

the center of the .sa3 output file.  The second time it

appears in the command line it is taken to be the Longitude

of the center of the .sa3 file.  The input format is

DDDdMM.mmmm

NOTE:

The program automatically performs a scan of all input points

in order to calculate a proper center and magnitude of .SA3 and

.SA4 output files.  If a latitude, longitude or magnification

is entered on the command line it will override the calculated

value.

g7to -i fname -o xxx.sa3 -g 12 -l N44d44.0 -l W122d45.0

This calculation is designed to return a magnification that will be

slightly smaller than might be shown on a single page, therefore,

all points should be displayed.

===========================================================================

Waypoint storage/retrieval accuracy.

The GPS45 sends/receives latitude/longitude data encoded in 4 byte

'long' variable.  However, the last byte of this variable is always

rounded by the GPS45.  Because of this there will be times when the

waypoint downloaded from the GPS45 will differ from that sent to it.

For instance if the waypoint:

W  TESTP2 N44 35.8931 ???????????? Mon Mar 25 15:28:04 1996 TEST WAYPOINT

is sent to the GPS45 it will be returned as

W  TESTP2 N44 35.8929 ???????????? Mon Mar 25 15:28:04 1996 TEST WAYPOINT

This 'rounding' applies to both the latitude and longitude variables.

===========================================================================

.TXT file format

The .txt file created by this program can contain Waypoint data,

proximity Waypoint data, Track data Route start, Route End, Lowrance

WSII file type Icon records and datum records.  This program assumes

WGS84 datum, and therefore the datum record is informational only, it

does not trigger a conversion.  The format for the various record types

is shown below.

Character positions are shown for clarity.

Route begin/end and Map datum records are stripped when the output is a

.sa3 file.

The datum for SA3/4 is WGS 84, and this program converts Lat/Lon data

literally, so the GPS must be using WGS 84/NAD83 or accuracy will be

degraded.

.TXT output format

Map datum:

111111111

123456789012345678

D  100	WGS 84

Waypoint, Proximity Waypoint (prox value starts at char position 78) and Track:

11111111112222222222333333333344444444445555555555666666666677777777778

12345678901234567890123456789012345678901234567890123456789012345678901234567890

N  New Track Start

W  YAQBRG N44 37.3259 W124 03.4229 Sun Mar 17 04:02:26 1996 YAQUINA BAY BRDG

P  YAQBRG N44 37.3259 W124 03.4229 Sun Mar 17 04:02:26 1996 YAQUINA BAY BRDG 234.5

T  N44 35.9589 W123 15.4792 Sun Jan 21 22:47:51 1996

! -b 12 -c oblack -c tblue -s ovs -s tm -a ne

Route begin:

11111111112222222222333333333344444444445555555555666666666677777777778

12345678901234567890123456789012345678901234567890123456789012345678901234567890

R  01 ROUTE NAME

Route end:

123456

E  01

===========================================================================

Proj format allowed on input (waypoint is output):

0123456789012345678921234567893123456789412345678951234567896123456789712345

124d04'36.28800"W 44d40'31.30200"N 1996-03-25 15:28 YAQHED YAQUINA HEAD

^           ^ ^

|           | |

must be 'd'    | |

must be '"'----. |

|

TAB or SPACE-----.

Proj format allowed on input (Track point is output):

0123456789012345678921234567893123

124d04'36.28800"W 44d40'31.30200"N

^           ^ ^

|           | |

must be 'd'    | |

must be '"'----. |

|

TAB or SPACE-----.

===========================================================================

Time:  If input is from a sa file, time is taken from the time stamp of

the input (-i) file.  For gardown7 or text input the time is

taken from the input record.  The GPS45 ignores the time stamp on

upload of track information.  The XL series does not store time,

therefore the time values on records from the XL receivers is not

meaningful.

===========================================================================

Distance:  The track distance calculation is *not* rigorous, it:

1.  Assumes that one minute of latitude is equal to 1 nautical mile.

2.  Assumes that longitude distance is related to latitude distance

by the Cosine function.

3.  Calculates the distance between two points by taking the

square root of the sum of the squares of the difference in

latitude and longitude.  Nautical miles are then converted to

statute mile by multiplying by 1.15078 and to kilometers by

multiplying by 1.852.

4.  Effects of SA are not removed.  A stationary track under SA

influence will cover quite a bit of ground.

5.  Distance is also calculated for Waypoints (but not Proximity

Waypoints), however as waypoints are usually stored in

alphabetical rather than chronological order the distance

calc is likely to be meaningless.

===========================================================================

.sa3 notes:  Street Atlas 3 has at least 3 objects that can be stored in

the .sa3 file.  These objects are Text, Place and MapNotes.

Places are very similar to waypoints, i.e.  there is a Lat/Long

location and a name.  All waypoints in a .txt file or from the

GPS45 become Places during a conversion to .sa3.

All three .sa3 objects are converted to waypoints during a .sa3

to GPS45 upload and/or a .txt conversion (and therefore would

become Places if the .txt file were used to create a new .sa3

file).

.sa4 notes:

Street Atlas 4 adds three more objects beyond those in sa3:  an

Area, a Circle and Lines.  The perimeter of an Area is converted

to track points by g7to.  The Circle is converted to proximity

waypoints and Lines are converted to tracks.

===========================================================================

Version History has moved to the file HISTORY.TXT

===========================================================================

A sample Almanac data file is shown between the 'Cut HERE' lines.

This sample file was created by

1.  Retrieve the data for week 851 from

http://satnav.atc.ll.mit.edu/gps/current.gpy and save it as file

current.gpy.

2.  Upload current.gpy to the GPS45 by: g45to -i current.gpy -o g45

3.  Download the sample file by g7to -i g45a >sample.fil

--------------------Cut HERE----------------------------

**** Week 910 almanac for PRN-01 ***********

ID:                          001

Health:                      000

Eccentricity:                3.856658936e-03

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.543541670e-01

Rate of Right Ascen(r/s):   -8.000333729e-09

SQRT(A) (m^1/2):             5.153687988e+03

Right Ascen at TOA(rad):    -1.886814356e+00

Argument of Perigee(rad):   -1.573585629e+00

Mean Anom(rad):             -2.690198660e+00

Af0(s):                      2.193450928e-05

Af1(s/s):                    0.000000000e+00

week:                        910

**** Week 910 almanac for PRN-02 ***********

ID:                          002

Health:                      000

Eccentricity:                1.675987244e-02

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.423220158e-01

Rate of Right Ascen(r/s):   -8.183198119e-09

SQRT(A) (m^1/2):             5.153627441e+03

Right Ascen at TOA(rad):     1.413419545e-01

Argument of Perigee(rad):   -2.362549782e+00

Mean Anom(rad):             -6.739662290e-01

Af0(s):                     -3.738403320e-04

Af1(s/s):                   -3.637978807e-12

week:                        910

**** Week 910 almanac for PRN-03 ***********

ID:                          003

Health:                      000

Eccentricity:                2.877712250e-03

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.525385499e-01

Rate of Right Ascen(r/s):   -8.217485359e-09

SQRT(A) (m^1/2):             5.153605469e+03

Right Ascen at TOA(rad):     1.219812036e+00

Argument of Perigee(rad):    2.512313843e+00

Mean Anom(rad):              1.897310972e+00

Af0(s):                      2.384185791e-05

Af1(s/s):                    3.637978807e-12

week:                        910

**** Week 910 almanac for PRN-04 ***********

ID:                          004

Health:                      000

Eccentricity:                4.290103912e-03

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.748052955e-01

Rate of Right Ascen(r/s):   -7.691749460e-09

SQRT(A) (m^1/2):             5.153489258e+03

Right Ascen at TOA(rad):     2.293904305e+00

Argument of Perigee(rad):   -8.844593763e-01

Mean Anom(rad):              1.606265903e+00

Af0(s):                      3.242492676e-05

Af1(s/s):                    0.000000000e+00

week:                        910

**** Week 910 almanac for PRN-05 ***********

ID:                          005

Health:                      000

Eccentricity:                1.026630402e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.451503158e-01

Rate of Right Ascen(r/s):   -8.114623640e-09

SQRT(A) (m^1/2):             5.153583008e+03

Right Ascen at TOA(rad):     1.570892185e-01

Argument of Perigee(rad):   -1.113309741e+00

Mean Anom(rad):              2.518942833e+00

Af0(s):                      1.001358032e-04

Af1(s/s):                    0.000000000e+00

week:                        910

**** Week 910 almanac for PRN-06 ***********

ID:                          006

Health:                      000

Eccentricity:                6.533145905e-03

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.595792890e-01

Rate of Right Ascen(r/s):   -8.137481800e-09

SQRT(A) (m^1/2):             5.153687988e+03

Right Ascen at TOA(rad):     1.253918767e+00

Argument of Perigee(rad):   -2.671363115e+00

Mean Anom(rad):              2.471879244e+00

Af0(s):                      0.000000000e+00

Af1(s/s):                    0.000000000e+00

week:                        910

**** Week 910 almanac for PRN-07 ***********

ID:                          007

Health:                      000

Eccentricity:                8.874416351e-03

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.619761705e-01

Rate of Right Ascen(r/s):   -8.103194560e-09

SQRT(A) (m^1/2):             5.153705078e+03

Right Ascen at TOA(rad):     1.220483541e+00

Argument of Perigee(rad):   -2.332727432e+00

Mean Anom(rad):             -1.894096255e+00

Af0(s):                      7.476806641e-04

Af1(s/s):                    0.000000000e+00

week:                        910

**** Week 910 almanac for PRN-09 ***********

ID:                          009

Health:                      000

Eccentricity:                6.117820740e-03

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.442934394e-01

Rate of Right Ascen(r/s):   -7.897472010e-09

SQRT(A) (m^1/2):             5.153591797e+03

Right Ascen at TOA(rad):    -8.685675859e-01

Argument of Perigee(rad):    1.722481400e-01

Mean Anom(rad):              2.388256788e+00

Af0(s):                     -9.536743164e-07

Af1(s/s):                    0.000000000e+00

week:                        910

**** Week 910 almanac for PRN-10 ***********

ID:                          010

Health:                      000

Eccentricity:                2.240657806e-03

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.635101557e-01

Rate of Right Ascen(r/s):   -7.897472010e-09

SQRT(A) (m^1/2):             5.153747559e+03

Right Ascen at TOA(rad):    -2.959578753e+00

Argument of Perigee(rad):   -3.029889166e-01

Mean Anom(rad):             -6.230965257e-01

Af0(s):                      9.536743164e-06

Af1(s/s):                    0.000000000e+00

week:                        910

**** Week 910 almanac for PRN-14 ***********

ID:                          014

Health:                      000

Eccentricity:                1.376152039e-03

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.689749479e-01

Rate of Right Ascen(r/s):   -7.886042930e-09

SQRT(A) (m^1/2):             5.153525391e+03

Right Ascen at TOA(rad):    -2.916636467e+00

Argument of Perigee(rad):    3.132910490e+00

Mean Anom(rad):             -1.832343102e+00

Af0(s):                      1.907348633e-05

Af1(s/s):                    0.000000000e+00

week:                        910

**** Week 910 almanac for PRN-15 ***********

ID:                          015

Health:                      000

Eccentricity:                7.090091705e-03

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.798206687e-01

Rate of Right Ascen(r/s):   -7.623174980e-09

SQRT(A) (m^1/2):             5.153542969e+03

Right Ascen at TOA(rad):     2.331382751e+00

Argument of Perigee(rad):    1.608242512e+00

Mean Anom(rad):              9.447830915e-01

Af0(s):                      4.367828369e-04

Af1(s/s):                    3.637978807e-12

week:                        910

**** Week 910 almanac for PRN-16 ***********

ID:                          016

Health:                      000

Eccentricity:                2.129554749e-03

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.653916359e-01

Rate of Right Ascen(r/s):   -7.908901090e-09

SQRT(A) (m^1/2):             5.153628906e+03

Right Ascen at TOA(rad):    -2.909928322e+00

Argument of Perigee(rad):   -1.007682309e-01

Mean Anom(rad):              8.457747102e-01

Af0(s):                      1.621246338e-05

Af1(s/s):                    0.000000000e+00

week:                        910

**** Week 910 almanac for PRN-17 ***********

ID:                          017

Health:                      000

Eccentricity:                9.355068207e-03

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.809890985e-01

Rate of Right Ascen(r/s):   -7.588887740e-09

SQRT(A) (m^1/2):             5.153604492e+03

Right Ascen at TOA(rad):     2.368193626e+00

Argument of Perigee(rad):    2.464549065e+00

Mean Anom(rad):              2.270262718e+00

Af0(s):                     -1.516342163e-04

Af1(s/s):                    0.000000000e+00

week:                        910

**** Week 910 almanac for PRN-18 ***********

ID:                          018

Health:                      000

Eccentricity:                6.513595581e-03

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.433166981e-01

Rate of Right Ascen(r/s):   -8.137481800e-09

SQRT(A) (m^1/2):             5.153595215e+03

Right Ascen at TOA(rad):    -1.942879200e+00

Argument of Perigee(rad):    1.628431439e+00

Mean Anom(rad):             -1.935205936e+00

Af0(s):                      9.536743164e-07

Af1(s/s):                    0.000000000e+00

week:                        910

**** Week 910 almanac for PRN-19 ***********

ID:                          019

Health:                      000

Eccentricity:                3.092288971e-03

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.277492166e-01

Rate of Right Ascen(r/s):   -8.023191889e-09

SQRT(A) (m^1/2):             5.153686523e+03

Right Ascen at TOA(rad):    -9.109883308e-01

Argument of Perigee(rad):   -2.928462267e+00

Mean Anom(rad):              1.628529906e+00

Af0(s):                      3.337860107e-05

Af1(s/s):                    2.910383046e-11

week:                        910

**** Week 910 almanac for PRN-21 ***********

ID:                          021

Health:                      000

Eccentricity:                1.339626312e-02

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.628390074e-01

Rate of Right Ascen(r/s):   -7.817468450e-09

SQRT(A) (m^1/2):             5.153619141e+03

Right Ascen at TOA(rad):    -2.954074383e+00

Argument of Perigee(rad):   -2.948176622e+00

Mean Anom(rad):             -2.036816329e-01

Af0(s):                      6.103515625e-05

Af1(s/s):                    3.637978807e-12

week:                        910

**** Week 910 almanac for PRN-22 ***********

ID:                          022

Health:                      000

Eccentricity:                9.831428528e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.423220158e-01

Rate of Right Ascen(r/s):   -8.137481800e-09

SQRT(A) (m^1/2):             5.153668945e+03

Right Ascen at TOA(rad):     1.552080661e-01

Argument of Perigee(rad):    2.002852410e-01

Mean Anom(rad):             -8.670500517e-01

Af0(s):                      3.976821899e-04

Af1(s/s):                    3.637978807e-12

week:                        910

**** Week 910 almanac for PRN-23 ***********

ID:                          023

Health:                      000

Eccentricity:                1.184749603e-02

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.660088420e-01

Rate of Right Ascen(r/s):   -7.897472010e-09

SQRT(A) (m^1/2):             5.153577637e+03

Right Ascen at TOA(rad):    -2.916052818e+00

Argument of Perigee(rad):   -2.116276741e+00

Mean Anom(rad):             -4.888144433e-01

Af0(s):                      9.536743164e-07

Af1(s/s):                    0.000000000e+00

week:                        910

**** Week 910 almanac for PRN-24 ***********

ID:                          024

Health:                      000

Eccentricity:                7.465839386e-03

Time of Applicability(s):    1.474560000e+05

Orbital Inclination(rad):    9.846383333e-01

Rate of Right Ascen(r/s):   -7.588887740e-09

SQRT(A) (m^1/2):             5.153632324e+03

Right Ascen at TOA(rad):     2.294521332e+00

Argument of Perigee(rad):   -1.892582178e+00

Mean Anom(rad):              2.107098579e+00

Af0(s):                      5.741119385e-04

Af1(s/s):                    3.274180926e-11

week:                        910

**** Week 910 almanac for PRN-25 ***********

ID:                          025

Health:                      000

Eccentricity:                6.579875946e-03

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.371808171e-01

Rate of Right Ascen(r/s):   -7.943188329e-09

SQRT(A) (m^1/2):             5.153763672e+03

Right Ascen at TOA(rad):    -9.004774094e-01

Argument of Perigee(rad):   -2.698281527e+00

Mean Anom(rad):             -2.909379482e+00

Af0(s):                     -5.722045898e-06

Af1(s/s):                    0.000000000e+00

week:                        910

**** Week 910 almanac for PRN-26 ***********

ID:                          026

Health:                      000

Eccentricity:                9.850978851e-03

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.573442340e-01

Rate of Right Ascen(r/s):   -7.966046489e-09

SQRT(A) (m^1/2):             5.153662109e+03

Right Ascen at TOA(rad):    -1.906025052e+00

Argument of Perigee(rad):   -4.097488821e-01

Mean Anom(rad):             -1.953893423e+00

Af0(s):                     -1.907348633e-04

Af1(s/s):                   -7.275957614e-12

week:                        910

**** Week 910 almanac for PRN-27 ***********

ID:                          027

Health:                      000

Eccentricity:                1.226186752e-02

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.413453341e-01

Rate of Right Ascen(r/s):   -7.874613850e-09

SQRT(A) (m^1/2):             5.153688965e+03

Right Ascen at TOA(rad):    -8.838508129e-01

Argument of Perigee(rad):    3.025349855e+00

Mean Anom(rad):              1.332262635e+00

Af0(s):                      3.242492676e-05

Af1(s/s):                    0.000000000e+00

week:                        910

**** Week 910 almanac for PRN-29 ***********

ID:                          029

Health:                      000

Eccentricity:                5.533218384e-03

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.537669420e-01

Rate of Right Ascen(r/s):   -8.023191889e-09

SQRT(A) (m^1/2):             5.153364746e+03

Right Ascen at TOA(rad):    -1.931037664e+00

Argument of Perigee(rad):   -1.923925400e+00

Mean Anom(rad):              2.868376732e+00

Af0(s):                      1.068115234e-04

Af1(s/s):                    5.093170330e-11

week:                        910

**** Week 910 almanac for PRN-30 ***********

ID:                          030

Health:                      000

Eccentricity:                5.172252655e-03

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.525625706e-01

Rate of Right Ascen(r/s):   -8.126052720e-09

SQRT(A) (m^1/2):             5.153703125e+03

Right Ascen at TOA(rad):     1.815239340e-01

Argument of Perigee(rad):    1.649238706e+00

Mean Anom(rad):             -7.610267401e-01

Af0(s):                     -6.198883057e-05

Af1(s/s):                   -3.637978807e-12

week:                        910

**** Week 910 almanac for PRN-31 ***********

ID:                          031

Health:                      000

Eccentricity:                7.439613342e-03

Time of Applicability(s):    6.144000000e+04

Orbital Inclination(rad):    9.614068866e-01

Rate of Right Ascen(r/s):   -8.137481800e-09

SQRT(A) (m^1/2):             5.153617188e+03

Right Ascen at TOA(rad):     1.222288370e+00

Argument of Perigee(rad):    7.284910679e-01

Mean Anom(rad):              3.117704630e+00

Af0(s):                      4.272460938e-04

Af1(s/s):                    2.546585165e-11

week:                        910

--------------------Cut HERE----------------------------

This program has been tested on several PC's running MS-DOS and/or

Windows 95.  It has also been tested on a Hewlett Packard 200LX palmtop.

The only Garmin unit available for testing this program is my own GPS45,

software revision 2.43, therefore the only upload/download functions in

this program are those known to be supported by the GPS45, Software

Version 2.43.

Versions of G7TO greater than 2.04 have been tested on my Garmin 12XL,

version 2.01, only.  I no longer own a GPS45.

Ron Henderson WA7TAS

crh@cv.hp.com