-----------------------------------------------------------------

Runs odd baud rates in doors with locked baud rates!

-----------------------------------------------------------------

Ever run into a door program that doesn't like your 14400 bps

or 16800 bps rate that is written to your DORINFOx.DEF? (Most notably

Imperium and Operation Overkill) Here's the fix! (see example for

Maximus specifically below)

-----------------------------------------------------------------

FIXINFO1 will change the baud rate written to DORINFO1.DEF to any

desired rate.

To use FIXINFO1.EXE type:

FIXINFO1 baud

baud is the locked port rate of your BBS.

Example:

FIXINFO1 19200

-----------------------------------------------------------------

FIXINFO2 will change the baud rate written to DORINFO2.DEF to any

desired rate.

To use FIXINFO2.EXE type:

FIXINFO2 baud

baud is the locked port rate of your BBS.

Example:

FIXINFO2 19200

-----------------------------------------------------------------

For multinode BBS's (3+) hex edit FIXINFO.EXE for proper DorinfoX.Def

(ie. change DORINFO1.DEF and DORINFO1.TMP to DORINFO3.DEF and DORINFO3.TMP

for node 3, etc)

----------------------------------------------------------------

Imperium Door Game Example for Maximus

----------------------------------------------------------------

1.  It is assumed you're using a locked baud rate via BNU or X00,

and have specified this baud rate in MAX.CTL as well as calling

MAX with "-s38400"

2.  Place FIXINFO1.EXE, FIXINFO2.EXE (if you are multi-node), BRUN45.EXE

in your \max directory.

3.  In MENUS.CTL:

Xtern_Erlvl    100_\Max\Imp\Imp.bat_%k    Normal "Imperium"

4.  In \max\imp\imp.bat:

C:

CD \MAX

Fixinfo%1 38400

cd \max\imp

Imperium \max\ DD %1

cd \max

----------------------------------------------------------------

That's all folks... note: No guarantee/warranty/etc/etc!

----------------------------------------------------------------