X00 is  a Serial Input/Output  (SIO) communications  driver.   It

provides  an interface  between  an application  program and  the

serial communications hardware.

X00 was  originally designed as  a FOSSIL  driver.  FOSSIL  is an

acronym containing the first  character of several communications

programs.   The FOSSIL specification is basically an extension of

the PC's BIOS services for serial communications.

Except as noted in  the Programmers Manual, X00 meets  the FOSSIL

level 5  specification.   X00 provides additional  functions that

are not contained in the FOSSIL level 5 specification.

X00 only works with 8250 (TYPE) serial I/O devices.  Such devices

include the 8250A,  16450, 16550, 16550A and  the 82510.   If you

have a PC that  is an IBM or  near compatible which has  a serial

communication  port, it is likely  that it contains  one of these

devices.  XU, a  utility distributed with X00, will  identify the

existence of compatible SIO devices.

Like  other  Device Drivers,  X00  will do  very  little standing

alone.  There must be an application(s) program to use X00 before

you will gain benefit.

Since  the introduction  of the  FOSSILs, a  great number  of new

communications  programs have  come into  existence.   I like  to

think  that the FOSSILs are  largely responsible for  this.  Many

programs that contain their own communications drivers have added

FOSSIL support.  One notable example is RBBS.

Many  programs  written  to  use  BIOS's   serial  communications

services will also work  with X00.  This gives those programs the

benefits of buffered interrupt driven I/O.

Installing X00

X00  is distributed  as a  ZIP file.   To extract  the individual

files  from the distribution file you need the program PKUNZIP or

an  equivalent.   PKUNZIP  is  a  product of  PKWARE  and  can be

downloaded from  almost any  bulletin board  system  (BBS).   You

probably already have PKUNZIP and know how to use it, otherwise I

doubt that  you would be reading  this.  When PKUNZIP  is used to

extract the various files, -AV should appear after each file name

as it is extracted.  In addition, the message:

Authentic files Verified!

should appear after the extraction process.  If this message does

not appear, or  appears with a name other than  Raymond L. Gwinn,

the file is not an original X00 distribution file.

The  only file  that the  basic user  actually needs  is X00.SYS.

Copy  this file to a convenient directory or subdirectory on your

system.   I suggest that all users  also install XU.EXE also.  XU

can provide important  information about X00 that  may be helpful

when problems occur.

Quick Start

Copy X00.SYS and XU.EXE to the root  directory of your boot drive

or diskette.

If you  are going  to use  standard communications  ports, either

COM1 or COM2 on  a PC or AT, or COM1 through COM8 on a PS/2, then

add the following line to your CONFIG.SYS file:

DEVICE=X00.SYS E

If  you wish  to have  more than  one communications  port active

simultaneously,  then  modify the  command  line  to include  the

number of simultaneously active  ports you wish to support.   For

example,  if you are using DESQview and you  wish to run a BBS in

two  different  windows,  then  add the  following  line  to your

CONFIG.SYS file:

DEVICE=X00.SYS E 2

It  should be  noted that the  above command  tells X00  how many

ports that are to  be used simultaneously, not which  ports.  The

application program may use ANY 2 serial ports in any order.

If your setup is more complex, you must read on.

Things  can get  a little confusing  here.   At the  time of this

writing,  X00 is almost (if not completely) unique in its ability

to function as a DOS device driver or as a Terminate (or Torment)

and Stay Resident program (TSR).

X00 loaded as a DOS Device Driver

If X00 is to be used as a DOS device driver, it must be installed

using  the CONFIG.SYS  file.   For a  precise description  of the

CONFIG.SYS file  and  its  function, please  refer  to  your  DOS

manual.  Basically,  you use a text  editor to add a  line to the

CONFIG.SYS  file.    The  line  that  you add  is  DEVICE=X00.SYS

followed by optional parameters.  The optional parameters will be

described later.  If X00.SYS is  not in the current directory you

will need to provide a DOS path  to the directory containing X00.

For  example, if X00.SYS is in  a subdirectory called COMM on the

hard  disk drive  C, the  line  in the  CONFIG.SYS file  would be

DEVICE=C:\COMM\X00.SYS.

X00 loaded as a TSR

If  X00 is  to be  used as  a Terminate  and Stay  Resident (TSR)

program,  then you must rename  or copy X00.SYS  to X00.EXE.  The

EXE  version of  X00 must  be in  the current  directory or  in a

directory  in the  DOS  path when  it is  executed.   X00.EXE  is

executed like  any other DOS program.   That is, you  type X00 at

the  DOS prompt followed by any optional parameters.  In reality,

you will probably be using a batch file to execute X00.

COMn and PORTn

X00 allows communications ports  to be swapped around.   That is,

the serial port that DOS calls COM1 can be mapped (reassigned) to

any X00 port.   Therefore, when a  reference in this  document is

made about  PORTn, it  refers to  the serial  I/O device  that is

mapped to  that X00  port.   When  a reference  is  made to  COM1

through  COM8,  it  refers to  the  serial  I/O  device that  DOS

knows/uses as COMn.   Also worth noting  is that DOS numbers  the

serial  I/0 devices  starting at  1 (COM1  through COM8)  and X00

numbers them starting at 0 (PORT0 through PORT7).

X00's  default is to assign  PORT0 through PORT7  to COM1 through

COM8.

COMMAND LINE OPTIONS

The  command line options are  included in the  command line that

invokes X00.  In several  cases only the first 1 or  2 characters

of the option need be entered.  In other cases, the entire option

name  must  be  spelled out.    Unless  otherwise  stated in  the

following descriptions,  the entire  option name must  be spelled

out.  The parameters are given one after another on the same line

and in any order.  The length of the command line is limited only

by DOS.

Specifying number of ports

If a  single  decimal number  is  in  the X00  command  line,  it

specifies  the number  of  ports that  X00  is to  simultaneously

support.   The default is  one port.   The maximum  value of  the

number of ports is 4 for non PS/2 systems and 8 for PS/2 systems.

If you  desire to use more than 4 ports  on a non PS/2, each port

address must  be specified (see mapping  port assignments below).

Input  and output  buffers are  allocated for  each of  the ports

specified.  Examples of usage are:

When X00 is loaded as a Device Driver

DEVICE=X00.SYS 3

DEVICE=X00.SYS 8

When X00 is loaded as a TSR

X00 3

X00 8

B option, Locking Baud, Data, Parity and Stop bits

Normally, the  application program sets  the baud rate  and other

parameters  controlling the  serial  I/O hardware.   Some  modems

provide  better  performance  when  the  communications  from the

computer to modem is constant (Locked).  That is, the computer to

modem speed is not the same as the modem to modem speed.  If this

option  is   used,  the  modem  must   support  RTS/CTS  hardware

handshaking and the feature must be enabled on the modem.

The B command  line option  requires 2 parameters  and allows  an

optional  third parameter.  The  option is specified  in the form

B,PORTn,BAUD  RATE,PARITY.  You  will most  likely never  want to

lock the parity, data and stop bits.  Examples of usage are:

When X00 is loaded as a Device Driver

DEVICE=X00.SYS B,0,2400 B,1,19200 or

DEVICE=X00.SYS B,1,19200,8N1

When X00 is loaded as a TSR

X00 B,0,2400 B,0,19200 or

X00 B,1,19200,8N1

Valid locked baud rates are: 75, 110, 135.5, 150, 300, 600, 1200,

1800, 2000,  2400, 3600, 4800,  7200, 9600, 19200,  28800, 38400,

57600, and 115200.

I can  think of few conditions  where it is necessary  to use the

optional  third  parameter of  the B  command  line option.   The

optional  third parameter is made up of 3 characters that specify

the  locked setting for data  bits per character,  parity bit and

stop bit(s) in  that order.  The optional  third parameter can be

made up of  any combination  of the following  first, second  and

third characters.

First     Second         Third

5         N = None       1

6         E = Even       2

7         O = Odd

8         M = Mark (always 1)

S = Space (always 0)

The first  character  specifies  the  number  of  data  bits  per

character,  the second  character specifies  the handling  of the

parity bit and the  third character specifies the number  of stop

bits.   When 5 data bits and  2 stop bits (eg 5N2) are specified,

the  number of stop bits will actually  be 1.5.  As stated above,

locking of the parity etc, is not advisable.

Anytime an application program attempts to program the baud rate,

parity etc, through X00, the locked values will be substituted.

Eliminate

Normally,  when X00 is loaded a copyright notice is displayed for

5 seconds.  Using the eliminate  option will disable the 5 second

display  of the copyright  notice.  Only the  first letter (E) is

required to invoke this option.  Examples of usage are:

When X00 is loaded as a Device Driver:

DEVICE=X00.SYS ELIMINATE or

DEVICE=X00.SYS E

When X00 is loaded as a TSR

X00 ELIMINATE or

X00 E

Defer

The Defer option  creates problems  on most systems.   The  Defer

option should be avoided by most users.  X00 must  use several of

the  80x86 interrupts.  Normally,  X00 will hook  itself into the

80x86 interrupts when it is executed.  The Defer option tells X00

to  postpone hooking  into some  interrupts until  an XU  CAPTURE

command  is  executed.    If  the defer  option  is  used  and  a

subsequent  XU  CAPTURE  is  not  issued,  X00  may not  function

correctly.  Additionally,  the XU CAPTURE must be  executed prior

to  executing  a multitasker  like DESQview.    Only the  first 2

letters (DE) need  to used to specify  this option.   Examples of

usage are:

When X00 is loaded as a Device Driver

DEVICE=X00.SYS DEFER or

DEVICE=X00.SYS DE

When X00 is loaded as a TSR

X00 DEFER or

X00 DE

The  default for  Defer is  off.   That is,  X00 will  by default

immediately hook into all necessary 80x86 interrupt vectors.

DV

The  DV command  line option is  only of interest  to those using

DESQview.   The  default for  this option  is off.   When  on (DV

specified  in the  command line),  X00 will  make DESQview  Pause

calls when it seems that  the application program is idle.   This

may make  time available for other  tasks to use.   Refer to your

DESQview manuals for details about the DESQview pause command.

If you are  using DESQview, try  the DV option.   If it does  not

adversely affect your system, leave it enabled (on).  Examples of

usage are:

When X00 is loaded as a Device Driver

DEVICE=X00.SYS DV

When X00 is loaded as a TSR

X00 DV

FIFO

Only the first  letter of the command  (F) is required to  invoke

this option.  Some of the serial I/O hardware  devices have First

In First Out (FIFO) buffers.  This command line option is used to

specify the  number of characters that X00 can load into the FIFO

during  a single  transmit  interrupt.   In  general this  number

should be set  the size of the  FIFO minus 1.   For example,  the

popular 16550A has a 16  byte FIFO on both transmit and  received

data.  Thus, if this option is used, I suggest the value of 15 be

used for the 16550A.  Examples of usage are:

When X00 is loaded as a Device Driver

DEVICE=X00.SYS FIFO=10

DEVICE=X00.SYS F=15

When X00 is loaded as a TSR

X00 FIFO=15

X00 F=8

The size of the receive FIFO is always set to the maximum by X00.

NOFIFO

This command line option is used to completely disable the use of

hardware FIFOs.  Examples of usage are:

When X00 is loaded as a Device Driver

DEVICE=X00.SYS NOFIFO

When X00 is loaded as a TSR

X00 NOFIFO

NOPOST

Normally,  when X00 is executed,  it will alter  the COM1 through

COM4 hardware port addresses  in BIOS RAM to match  the addresses

being used by X00.  By doing so, many DOOR type programs that use

INT  14h  for  their serial  I/O  will  work  correctly with  X00

installed.   This option (NOPOST) instructs X00 to leave the BIOS

RAM unchanged.  Examples of usage are:

When X00 is loaded as a Device Driver

DEVICE=X00.SYS NOPOST

When X00 is loaded as a TSR

X00 NOPOST

SLOWCPU

This command line option  is provided for those using  slower PCs

and those using software  that executes large amounts of  code on

an interrupt  level (such  as the timer  tick).  When  SLOWCPU is

specified,  X00 will make  the serial ports  the highest priority

interrupt.     This  feature  allows   (most)  4.77  mhz  PCs  to

receive/transmit at full  speed.  This option  may cause problems

in  some  environments.    Most  protected  mode  programs,  like

DESQview, will not  allow the  priority of the  interrupts to  be

changed.

When X00 is loaded as a Device Driver

DEVICE=X00.SYS SLOWCPU

When X00 is loaded as a TSR

X00 SLOWCPU

R, Set Receive Buffer Size

The R command line option allows the user to  set the size of the

receive buffer.   The default size  of the receive buffer  is 512

bytes.   The  size of  the buffer must  be a  power of  2.  Valid

buffer sizes are:  256, 512,  1024, 2048, 4096,  8192, 16384  and

32768.  If an  invalid buffer size  is specified, the next  lower

correct buffer size is used.

Many users needlessly set  the size of the receive  (or transmit)

buffer too large.  Examples of usage are:

When X00 is loaded as a Device Driver

DEVICE=X00.SYS R=1024

DEVICE=X00.SYS R=2048

When X00 is loaded as a TSR

X00 R=1024

X00 R=2048

T, Set Transmit Buffer Size

This option is identical to the R command line option (see above)

except it sets the size of the transmit buffer.

VP or Master Interrupt Enable

VP stands for Vector  Port which is not a very good  name for the

option.  Some multiple port serial I/O cards default to  only two

ports enabled when power is turned on.  A specified value must be

written to an  I/O port  to enable the  additional serial  ports.

Simply stated the VP command line  option says to write the given

value to the given port.  Examples of usage are:

When X00 is loaded as a Device Driver

DEVICE=X00.SYS VP=03FF,80

When X00 is loaded as a TSR

X00 VP=3FF,80

Both of  the above examples  in effect says,  write the value  80

(hex) the I/O port at 3FF.

Mapping port assignments

This  is the most complicated command line option for many users.

By default,  X00 maps up to  8 serial I/O ports  (PORTS 0 through

7).  X00 will (attempt to) determine the hardware architecture on

which  it is  being executed.   If  a PS/2  is detected  then the

default Serial  I/O port addresses will be  set to match the PS/2

(see Appendix  .  If X00  detects that the  system is not  a PS/2

type system, then the default port addresses are as follows:  1 -

Standard  addresses for COM1 and  COM2, 2 -  The defacto standard

for COM3 and  COM4, and nothing  for COM5  through COM8 (ports  4

through 7).

Port mapping is necessary if your hardware configuration does not

match the PS/2 or  defacto standard for non  PS/2 systems.   COM1

and COM2  are the same  for the PC,  XT, AT  and PS/2.   However,

hardware  specifications for COM3  through COM8  became available

only when IBM released the PS/2.

Many multiple  serial  port  cards are  available  for  pre  PS/2

systems that use a defacto standard.  The most common assignments

that I have seen for COM3 and COM4 are:  COM3 at port address 3E8

using IRQ 3.   COM4 at port  address 2E8 using IRQ 4.   Above the

fourth port, I  have not observed  anything resembling a  defacto

standard.

X00  will support almost any  serial I/O card.   However, in most

cases,  port mapping  is required  for anything  beyond COM1  and

COM2.

Mapping port assignments uses two basic forms.  They are:

PORTn=COMn,IRQn

PORTn=HEXADDRESS,IRQn

Where:

PORTn is the X00 port number and must be 0 through 7.

COMn is a standard DOS port and must be COM1 through COM8.

IRQn is the IRQ use by the serial port and must be IRQ0

through IRQ15.

HEXADDRESS is a hexadecimal I/O port address that addresses

the serial I/O device.

In both basic forms, the IRQn is optional.  X00 defaults all IRQn

to  IRQ3 except COM1  which defaults to IRQ4.   Examples of usage

are:

When X00 is loaded as a Device Driver

DEVICE=X00.SYS 0=COM4

DEVICE=X00.SYS 0=COM4,IRQ5

DEVICE=X00.SYS 0=2E8,IRQ3

DEVICE=X00.SYS 2=2E8

When X00 is loaded as a TSR

X00 0=COM4

X00 0=COM4,IRQ5

X00 0=2E8,IRQ3

X00 2=2E8

Un-installing X00

When X00  is installed as a TSR, it can be un-installed by simply

executing X00 with no command line parameters.  If you wish to be

sure that you do not accidently un-install X00, always execute it

with a harmless parameter like X00 E.