RTSP Programmer’s Guide

Jim A. Ledin, PE

August 24, 2000

# Overview

The Real Time Simulation Protocol (RTSP) is a package of C++ software tools and techniques that enable high performance real time distributed simulation across a TCP/IP network. The RTSP design has two primary goals: Maximum real time performance and ease of use across a variety of computing platforms. The goal of maximizing real time performance is satisfied with the use of the UDP multicast protocol and by performing as much of the work as possible during the compilation and initialization phases, thus minimizing the computation required during real time operation. Ease of use is provided by the Message Definition File (MDF) format to specify the contents of data messages and which simulations will receive each message type. Other techniques that increase the ease of use are the automatic C++ code generation by the MDF translator as well as automatic handling of cross-platform issues such as byte ordering and the prevention of "holes" in data structures.

We will adopt the High Level Architecture (HLA) terminology for distributed simulations and call an entire distributed simulation a federation and each individual simulation a federate. The term federate is actually more general than just referring to a simulation. A federate can also be a human-in-the-loop simulator or it could be an actual embedded system that is interfaced as part of a distributed simulation. An example of this type of federate is a radar console that can display simulated aircraft tracks while operating in a simulation mode.

The use of UDP multicast minimizes the amount of network traffic required since each message only needs to be transmitted one time but can be received by any number of federates. The UDP protocol is unreliable, which means that if a data packet gets lost or corrupted while traveling across the network it will be gone. Lost packets are not retransmitted automatically. This is actually a good thing for our real time application because the packet loss detection and retransmission mechanisms used by other protocols such as TCP result in a lengthy process that blocks delivery of subsequent data until retransmission has been completed. With UDP, we simply coast over a lost packet and continue with the next packet that arrives without any additional delay. Of course, this means that we must transmit updates frequently enough that the loss of a single update does not significantly degrade any simulation's operation. RTSP keeps track of lost packets so that at the end of a simulation run it is possible to determine when data dropouts occurred and how long they lasted.

For maximum simulation fidelity, it is desirable to minimize the effects of communication latency by extrapolating the value of continuous variables to the current time in the receiving application. This presents a problem because some method must be available for determining the elapsed time between the initiation of message transmission and the current time. If the federates are all running on the same computer, this is a trivial problem, since all federates use the same system clock. However, if the federates are running on separate computers (possibly thousands of miles apart) it may be difficult or impossible to synchronize computer system clocks to the level required level (milliseconds or microseconds). Some other approach must be used. One way to do this is to install a Global Positioning System (GPS) receiver interface in each computer in the federation. As a byproduct of the GPS position measurement, the receiver determines the precise time. GPS interface cards can provide the current time with accuracy in the single-digit microsecond range. RTSP supports the use of GPS time (or other precision time reference) to measure communication latency for use in data extrapolation.

The RTSP runtime software also provides the capability to log all messages received by a federate to disk as they arrive. Time of receipt for each message is also stored in the log file so that detailed studies of communication latency can be performed during post-run analysis. Of course, this message logging facility will detract from the real time performance of a simulation, so in many cases it makes more sense to create a separate federate that does nothing but receive network traffic and logs it to disk. This federate places no additional burden on the real time network communication as long as it does not transmit any messages.

The Message Definition File (MDF) format provided with the RTSP enables the development of a text file that completely describes the data messages transferred across the network and which of the federates send and receive each type of message. The MDF syntax is similar to the C language elements used to describe data structures. An MDF translator program reads the MDF file and converts it into a set of C++ header files and a single C++ source file containing static data describing the federation. The simulation developer uses these files along with the provided RTSP runtime source code to build the member federates of the federation.

A generic Win32 federation controller program is also provided with the RTSP distribution. This program reads the federation MDF file and provides a set of buttons that allow the user to send command messages to the federates. The controller displays the response received from each federate following each transmission of a command message. The available commands are:

- Ping: Time how long each federate takes to respond, if it responds at all.

- Initialize: Command all federates to prepare for a simulation run.

- Start: Start real time execution immediately.

- Halt: Stop real time execution immediately.

The RTSP software has been tested on several different platforms including PCs (Windows 98 and Windows NT 4.0), Unix (Solaris), and a real time operating system (VxWorks on a PowerPC processor). RTSP system requirements include a standard TCP/IP programming interface that includes multicast support as well as an interface to a network that provides multicast support. The use of a precision time reference such as a GPS receiver is optional.

On a low-traffic local area network using PCs and standard Ethernet hardware, one-way communication latencies of 1 millisecond are typical. RTSP also makes efficient use of system resources: On a PC running three simulations where each simulation sends 100 updates/second the CPU usage (as indicated by the Windows NT Task Manager) is only in the 1-2% range.

The remainder of this document describes the components of RTSP in detail and provides the necessary instructions for using them. A complete distributed simulation consisting of three federates is also described.

# RTSP Components

The RTSP package includes four primary components that work together to enable networked simulations.

## Message Definition Format File

The Message Definition Format (MDF) file defines the data messages that pass among the federates and also specifies which federates can send and receive which message types. The MDF file can optionally include parameters that specify the base values for the range of multicast network addresses used, the base TCP/IP port number for the range of ports used, and the multicast time-to-live (TTL) value that specifies how many routers the multicast traffic is allowed to cross.

The MDF file format is described in detail in the section "Message Definition File Format Reference" below.

## MDF Translator

The MDF translator program converts an MDF file into a set of C++ header files and a single C++ source file containing static data that describes the federation.

Instructions for running the translator appear in the "Software Tools" section below.

## RTSP Runtime Software

RTSP federates must use the RTSP.h and RTSP.cpp source files, in addition to the source files generated by the MDF translator program. The RTSP.h and RTSP.cpp files contain the definition of the RTSP class plus some type definitions used within the RTSP software.

The type definitions and members of the RTSP class are described in the "RTSP Class" section below.

## Generic Controller Application

The generic federation controller program is also provided with the RTSP distribution. This Win32 program reads the federation MDF file and provides a set of buttons that allow the user to send command messages to the federates. The controller displays the response received from each federate following each transmission of a command message.

Instructions for using the generic controller appear in the "Software Tools" section below.

# RTSP Class

The RTSP class is implemented in the files RTSP.h, RTSP_Internals.h, and RTSP.cpp. For normal applications the contents of the RTSP_Internals.h file are irrelevant, so that file will not be discussed here.

The RTSP.h file contains some type definitions and the definition for the RTSP class.

## Type Definitions

The bool enumeration type definition should only be used if you have an older C++ compiler that doesn't implement the built-in bool type. If you have a newer compiler that does provide bool support, comment out or delete this type definition.

enum bool { false, true }; // For a compiler that doesn't support bool type

The other type definitions are simply shorthand versions of unsigned types. Since these are common type definitions, they may conflict with other type definitions in your code. If a conflict occurs, it is acceptable to comment out or delete these type definitions as long as the type names do not change meaning.

typedef unsigned char  uchar;

typedef unsigned short ushort;

typedef unsigned int   uint;

## RTSP Class Type Definitions

The RTSP class StateType is an enumeration that lists the possible execution states a federate can be in.

enum StateType

{

Stopped,

Initialized,

Running

};

The RTSP class MsgType is an enumeration that lists the possible command message types that a controller can send.

enum MsgType

{

Ping,

Initialize,

Start,

Halt

};

The RTSP class LostMsgRecord is a structure that contains information about a sequence of lost messages. The members in this structure identify the message index number as defined in the _MsgData.cpp file, the federate indexes of the sender and receiver, the sequence numbers of the first and last lost messages, and the time tag at which the lost message sequence was detected.

struct LostMsgRecord

{

uint msg_index;

uchar send_index;

uchar rcv_index;

ushort lost_seq_start;

ushort lost_seq_end;

uint time_tag;

};

The RTSP class m_LostMsgLogSize is an enumeration constant used to size the array of LostMsgRecord structures. The value of this enumeration constant should be selected to balance the needs to minimize memory use while allowing the necessary amount of data collection about sequences of lost messages. The default value enables collection of information about 50 sequences of lost messages during a simulation run.

enum { m_LostMsgLogSize = 50 };

The RTSP class MsgLogHeader contains additional information that is written to the log file along with the contents of arriving messages if logging to disk is enabled.

struct MsgLogHeader

{

ushort rec_size;    // total size of header + message in bytes

ushort port;        // IP port number for this message

uint log_time_tag;  // Time tag when message received

};

## Constructors and Destructor

Only federation controllers can use the first constructor shown below. It sets up TCP/IP for communication as a controller.

RTSP();

Only non-controller federates can use the second constructor shown below. It sets up TCP/IP for communication as the named federate, which must exactly match the name of a federate given in the MDF file for the federation. The fed_index argument must be zero (the default) if the federate declaration in the MDF file is not an array type. If the federate declaration is an array type, each federate in the array must have a unique fed_index value starting from 0 up to the array length minus one.

RTSP(const char* fed_name, uint fed_index=0);

The desctructor for the RTSP class closes the sockets used for communication.

~RTSP();

Message Logging Control

The RTSP class EnableMessageLogging member function creates a file with a name in the format "YYMMDD-HHMMSS.log", where YYMMDD represents the year, month and day and HHMMSS represents the current hour, minute and second. Subsequent messages received by the federate will be logged in binary format to the file with a header of type MsgLogHeader for each one. Message logging will continue until a call to the DisableMessageLogging member function is executed.

On initialization of the RTSP class, message logging is disabled. Whenever a Halt message arrives, if message logging is enable the current message log file is closed and a new message log file is opened by calling the EnableMessageLogging member function. The result of this is that each simulation run will be stored in a separate log file with the time and date of the end of the previous run indicated in the log file name.

void EnableMessageLogging();

The RTSP class DisableMessageLogging member stops the logging of messages.

void DisableMessageLogging();

RTSP Configuration Member Functions

The RTSP class SetInterfaceAddr member function allows the selection of which network interface to for RTSP communication in a computer with more than one network interface.

void SetInterfaceAddr(const char* addr);

The RTSP class SetEndianness member function allows the override of the automatically detected byte ordering determined during the RTSP constructor execution. This capability would be useful in the situation where one processor is performing RTSP communication and the RTSP message data is passed directly to and from another processor with opposite byte ordering that runs the actual simulation.

void SetEndianness(bool is_big_endian);

RTSP Incoming Message Processing Member Functions

The PollRcv member function waits for an incoming message to arrive or the timeout interval to elapse before returning. If a message arrives while PollRcv is executing, the message data will be placed into the associated C++ object and the message callback function will execute.

If the use_timeout parameter is true, the PollRcv function will wait the number of microseconds indicted by the timeout_usec parameter before returning. If use_timeout is true and timeout_usec is zero, the PollRcv function will process any incoming messages that have already arrived and are awaiting delivery before returning.

If the use_timeout parameter is false, PollRcv will wait indefinitely for an incoming message to arrive before processing it and returning.

void PollRcv(bool use_timeout, uint timeout_usec=0);

The RcvMainLoop member function is intended for when separate receive and transmit threads are in operation. This function blocks awaiting the arrival of incoming messages, then processes each one as it arrives. RcvMainLoop never returns.

void RcvMainLoop(void);

RTSP Timer and Clock Member Functions

The RTSP timer counts microseconds and returns a 32 bit unsigned integer that indicates the elapsed time since the timer started. The StartTimer member function starts the timer at zero counting upward.

void StartTimer(void);

The StopTimer member function freezes the timer at the current value.

void StopTimer(void);

The ReadTimer member function returns the current timer value in microseconds since timer start, or zero if the timer has not been started, or the stop time if it has been started and then stopped. The timer will wrap back to zero after the 32 bit count overflows, which occurs about every 72 minutes.

uint ReadTimer(void);

The ReadClock member function returns the current system clock time in microseconds past the hour. The largest possible value ReadClock can return is 3.6e9 – 1. This function is used internally by the RTSP software to as the message time tag values.

uint ReadClock(void);

The AddTime member function adds two clock values in microseconds past the hour while accounting for the hourly reset to zero.

uint AddTime(uint time1, uint time2);

The DeltaTime member function subtracts two clock values in microseconds past the hour while accounting for the hourly reset to zero. If the earlier time value is larger than the later time value, DeltaTime assumes that the earlier time was before the hour and the later time was after the hour.

uint DeltaTime(uint earlier_time, uint later_time);

RTSP Federation Controller Member Functions

The CmdFederation member function is used by federation controllers to issue command messages. The cmd parameter is of the type MsgType described above in the "Type Definitions" section. The timeout_secs parameter indicates the time in seconds that is allowed for responses to arrive at the controller from each federate. The return value is true if all federates respond within timeout_secs and all federates return a status indication of success. Otherwise, the return value is false.

bool CmdFederation(MsgType cmd, double timeout_secs=2.0);

RTSP Federate Member Functions

The WaitForInit member function is used by federates to pause until an Initialization command message arrives from the federation controller. Incoming messages are received and processed and callback functions are executed by the WaitForInit function until an Initialization command message arrives. The function then returns.

void WaitForInit(void);

The WaitForStart member function is used by federates to pause until a Start command message arrives from the federation controller. Incoming messages are received and processed and callback functions are executed by the WaitForStart function until a Start command message arrives. The function then returns.

void WaitForStart(void);

The WaitForHalt member function is used by federates to pause until a Halt command message arrives from the federation controller. Incoming messages are received and processed and callback functions are executed by the WaitForHalt function until a Halt command message arrives. The function then returns.

void WaitForHalt(void);

The RequestHalt member function is used by federates to request that the controller send a Halt message to the federation. This function is called by a federate when a condition occurs that should stop the federation execution.

void RequestHalt(void);

RTSP Error Processing Member Function

The Error member function displays the error message given in the msg parameter on the stderr device and optionally exits the program if the abort parameter is true. If abort is false, execution continues normally.

static void Error(bool abort, const char* msg);

RTSP Federate and Federation Data Access Member Functions

The IsController member function returns true if the application is a controller or false if it is a federate. The type of RTSP constructor used determines whether an application is or is not a controller.

bool IsController(void) const;

The FederateIndex member function returns the index of the federate. This will be zero for a non-array federate, or it will be the index assigned in the RTSP constructor for an array federate.

uchar FederateIndex(void) const;

The IsBigEndian member function returns true if the big-endian byte ordering is used or false if little-endian ordering is used.

static bool IsBigEndian(void);

The InterfaceAddr member function returns the IP address of the network interface used by RTSP as a character string in dotted-decimal notation.

static const char* InterfaceAddr(void);

The FederateState member function returns the execution state of the federate using the StateType definition described above in "Type Definitions". This function can only be called by federates.

StateType FederateState();

The FederationState member function returns the execution state of the federation using the StateType definition described above in "Type Definitions". This function can only be called by controllers.

StateType FederationState();

RTSP Federate Callback Setup Member Functions

These functions are used by federates to set up callback functions to be executed when particular command messages arrive at the federate. The callback functions must be of the form:

bool f()

{

bool result;

// Perform any needed processing here

// Set result = true if successful, false if not

return result;

}

The parameter for any of the following functions would be the name of this function, f. If any federate returns false in response to an Initialize, Start, or Halt command, the federation state does not change to the requested state. For proper RTSP operation, all federates must respond with success indications to these command messages.

These callback routines are optional. If any callback routine is not provided, the RTSP software will automatically respond with a success indication to each of these messages.

void SetPingCallback(bool (*f)(void));

void SetInitializeCallback(bool (*f)(void));

void SetStartCallback(bool (*f)(void));

void SetHaltCallback(bool (*f)(void));

RTSP Lost Message Handling Member Functions

The NumLosgMsgs member function returns an integer value indicating how many sequences of lost messages have been detected during the current simulation run, up to the maximum value indicated in the enumeration constant m_LostMsgLogSize.

uint NumLostMsgs();

The GetLostMsgRecord member function returns one record indicating a sequence of lost messages at the index provided as a parameter. The index parameter must be in the range 0 to the value returned by NumLostMsgs()-1.

LostMsgRecord GetLostMsgRecord(uint index);

RTSP Miscellaneous Member Function

The AddSocketToSet member function is used internally by the RTSP software, but must be globally declared in the RTSP class. In normal applications, this function should not be called by controllers or federates.

void AddSocketToSet(uint fd) { FD_SET(fd, &m_SocketSet); }

# Message Definition Format File Reference

An MDF file contains a series of statements, which can be arranged in a somewhat arbitrary order. Each statement ends in a semicolon.. Many of the same rules that apply to the C and C++ languages also apply to the MDF file format. For instance, keywords and names are case-sensitive, and statements are free-format, which means that they can stretch over several lines if desired.

Comments may be placed in the MDF file using C format ("/* … */") or C++ format ("// …").

The following sections define each of the possible MDF statements.

## Multicast Base Address

This parameter defines the base IP address used for the multicast message traffic. Each type of message used in RTSP is assigned a unique multicast address that consists of this base plus an offset value. This addition operation treats the base address as an unsigned 32-bit integer with the most significant byte at the left. Two multicast addresses are reserved for command messages: multicast_base_addr+0 is used for command messages and multicast_base_addr+1 is used for command message responses.

Each message defined in the MDF file is assigned a unique offset value starting with 2. After running the MDF translator program, you can determine the IP address for each message type by searching the generated _MsgData.cpp file for "_MsgContentsList" and finding the line within this structure containing the name of the message of interest.

The syntax of the multicast_base_addr statement is:

multicast_base_addr = NUMBER.NUMBER.NUMBER.NUMBER;

In this definition, NUMBER is a decimal integer in the range 0 to 255. These values represent a valid IP multicast address in the range 224.0.0.0 to 239.255.255.255. In addition, this base address must be low enough that the highest offset added to it as a result of processing the MDF message definition does not exceed the upper multicast limit of 239.255.255.255.

Here is an example of a valid multicast_base_addr statement:

multicast_base_addr = 225.0.1.100;

If the address provided in this statement is not in the permitted range, the translator program will issue an error message. If no multicast_base_addr statement appears in an MDF file, the default base address of 225.0.0.0 will be used. This statement can appear anywhere in the MDF file. By convention, it is placed at the top of the file before any federate or message definitions.

## Multicast Base Port

This parameter defines the base IP port number used for the multicast message traffic. Each type of message used in RTSP is assigned a unique multicast port number that consists of this base plus an offset value. Two multicast port numbers are reserved for command messages: multicast_base_port+0 is used for command messages and multicast_base_port+1 is used for command message responses.

Each message defined in the MDF file is assigned a unique offset value starting with 2. After running the MDF translator program, you can determine the port number for each message type by searching the generated _MsgData.cpp file for "_MsgContentsList" and finding the line within this structure containing the name of the message of interest.

The syntax of the multicast_base_port statement is:

multicast_base_port = NUMBER;

In this definition, NUMBER is a positive decimal integer that is a valid, available IP port number. There must be a range of available IP port numbers starting with the base port number large enough to accommodate the two ports used by command messages plus the number of messages defined in the MDF file.

Here is an example of a valid multicast_base_port statement:

multicast_base_port = 12000;

If no multicast_base_port statement appears in an MDF file, the default base port number of 20000 will be used. This statement can appear anywhere in the MDF file. By convention, it is placed at the top of the file before any federate or message definitions.

## Multicast Time-to-Live (TTL)

This parameter defines the number of routers that RTSP multicast messages can pass through before the messages are deleted from the network. If you are running RTSP on a large network with many users, you should make the TTL as small as possible to minimize the burden placed on the network by your multicast traffic.

The syntax of the multicast_time_to_live statement is:

multicast_time_to_live = NUMBER;

In this definition, NUMBER is a decimal integer in the range 1 to 255.

This is an example of a valid multicast_time_to_live statement:

multicast_time_to_live = 3;

If no multicast_time_to_live statement appears in an MDF file, the default value of 1 will be used. This statement can appear anywhere in the MDF file. By convention, it is placed at the top of the file before any federate or message definitions.

## Constant Declaration

The constant declaration statement assigns an integer value to a symbol. This symbol can be used at any point later in the file where an integer value is required, such as in defining the dimension of an array of federates.

The syntax for the constant declaration is:

constant SYMBOL = NUMBER;

In this definition, SYMBOL is a name starting with an uppercase or lowercase letter followed by any combination of letters, numbers, and underscores. Number is a positive integer.

This is an example of a valid constant declaration:

constant NumTargets = 2;

Constant declarations are optional and serve only to clarify the contents of MDF files. A constant declaration can appear anywhere in the MDF file, but it must occur above the point where the symbol is first used in the file.

## Federate Declaration

The federate declaration defines the names of one or more federates in a simulation and also indicates the number of identical federates of a particular type in use, if there are more than one.

The syntax of the federate declaration is as follows. The square brackets and ellipses represent the optional addition of federate names.

federate NAME [, NAME, …];

In this definition, NAME is a federate name starting with an uppercase or lowercase letter followed by any combination of letters, numbers, and underscores. Each NAME optionally includes a dimension in the form of an integer (or constant symbol representing an integer) in square brackets. Any number of federates can be declared in a single federate statement. Equivalently, any number of federate statements may appear in an MDF file.

The following example declares four federate names and indicates that there will be two identical Target federates. This assumes that the constant declaration example from the previous section appears earlier in the MDF file.

federate Launcher, Missile, Target[NumTargets], Viewer;

A federate declaration can appear anywhere in the MDF file, but it must occur above the point where the federate name is used in a message definition.

## Structure Declaration

A data structure declaration creates a definition of a sub-structure that can be used in subsequent structure and message declarations. The syntax of a structure declaration is similar to the struct statement in C and C++, with some limitations.

In MDF files, the struct definition can contain previously declared structs or basic RTSP data types. Any element in a struct can be declared as a one-dimensional array, if desired. The basic types available in RTSP are char, uchar, short, ushort, int, uint, float, and double. The basic types that start with "u" are unsigned data types.

RTSP requires that char and uchar are 1 byte long;  short and ushort are 2 bytes long; int, uint, and  float are 4 bytes long and double is 8 bytes long. During initialization, the run time RTSP software verifies that these conditions are satisfied. If they are not, the software reports an error and exits.

The format of the structure declaration is the keyword struct followed by the structure name, followed by a list of structure elements inside curly braces. Like other MDF statements, the struct statement is terminated with a semicolon. The syntax of the struct statement is:

struct NAME { list_of_declarations };

In this definition, NAME is a structure name starting with an uppercase or lowercase letter followed by any combination of letters, numbers, and underscores.  list_of_declarations represents a series of one or more structure element declaration statements.

Each structure element declaration statement consists of a basic type name or already-defined structure name followed by a comma-separated list of structure element names. A dimension in square brackets can optionally follow each structure element name. Each structure element declaration must terminate with a semicolon.

This is an example of a valid structure definition.

struct State

{

double altitude, pos[3];

double vel[3];

};

A structure declaration can appear anywhere in the MDF file, but it must occur above the point where the structure is used in a message definition or another structure definition.

## Message Declaration

A message declaration creates a definition of a data structure that is transferred among the federates during execution. The syntax of a structure declaration is similar to the struct statement in C and C++, with some limitations and differences.

In MDF files, the message definition can contain previously declared structs or basic RTSP data types. Any element in a message can be declared as a one-dimensional array, if desired. The basic types available in RTSP are char, uchar, short, ushort, int, uint, float, and double. The basic types that start with "u" are unsigned data types.

RTSP requires that char and uchar are 1 byte long;  short and ushort are 2 bytes long; int, uint, and  float are 4 bytes long and double is 8 bytes long. During initialization, the run time RTSP software verifies that these conditions are satisfied. If they are not, the software reports an error and exits.

The format of the message declaration is the keyword message followed by the name of the federate that transmits the message, then a period, and then the message name. This is followed by a list of structure elements inside curly braces. Like other MDF statements, the struct statement is terminated with a semicolon. The syntax of the struct statement is:

message FEDERATE_NAME.MESSAGE_NAME { list_of_declarations };

In this definition, FEDERATE_NAME is the name of an already-defined federate and MESSAGE_NAME is a name starting with an uppercase or lowercase letter followed by any combination of letters, numbers, and underscores.  list_of_declarations represents a series of one or more message element declaration statements.

Each message element declaration statement consists of a basic type name or already-defined structure name followed by a comma-separated list of message element names. A dimension in square brackets can optionally follow each structure element name. Each structure element declaration must terminate with a semicolon.

This is an example of a valid message definition.

message Launcher.Update

{

State state;

uchar status[2];

};

A message declaration can appear anywhere in the MDF file, but it must occur after the point where the sending federate and any structures used in the message are declared.

## Message Subscription Declaration

Each federate can subscribe to (receive) any message in the MDF file, including ones that it transmits itself. The subscription declaration defines the list of message types that each federate wishes to receive. For messages that are sent by federates that are declared in an array, the subscribing federates will receive messages from every federate in the array. If the subscribing federate name is an array of federates, all federates in the array will receive the messages.

The syntax of the message subscription declaration is as follows:

subscribe SUBSCRIBING_FEDERATE_NAME : FEDERATE_NAME.MESSAGE_NAME;

In this definition, SUBSCRIBING_FEDERATE_NAME is the name of the federate that will receive the messages, FEDERATE_NAME is the name of the federate that transmits the messages, and MESSAGE_NAME is the name of the message that is being subscribed to.

The message name can be given as an asterisk, which means that all messages transmitted by the given federate will be received. The sending federate name can also be given as an asterisk, meaning that messages from all federates will be received. In this case, the message name must also be an asterisk.

The examples below demonstrate some valid message subscription declarations:

// The Launcher gets updates from all Targets

subscribe Launcher : Target.Update;

// The Viewer gets all Update messages sent by all federates

subscribe Viewer : *.*;

A message subscription declaration can appear anywhere in the MDF file, but it must occur after the point where the federates and messages that appear in it have been declared.

# Software Tools

## MDF File Translator

The MDF file translator is a Win32 application that reads as input an MDF file and generates a set of C++ header files and a single C++ source file containing static data definitions. The name of the application is translator.exe and the syntax for running it is:

translator file_name.mdf

In this definition, file_name.mdf represents the name of an existing MDF file.

If errors occur during the execution of the translator, error messages will be displayed on the screen indicating the cause of the problem and their location in the MDF file.

During successful execution, the translator will display the names of the output C++ files generated. This is the output from a successful run of the translator:

C:\Projects\DistSim>translator DistSim.mdf

Generating CommandGeneratorSend.h

Generating ControllerSend.h

Generating ControllerRcv.h

Generating PlantSend.h

Generating PlantRcv.h

Generating _MsgData.cpp

C:\Projects\DistSim>

### Generated Code Format

The code generated by the MDF translator creates a group of C++ header files with a similar format. The contents of the PlantSend.h file are:

// PlantSend class

// Generated Thu Aug 24 13:33:36 2000 from distsim.mdf

// *** This file was automatically generated. Do not edit it! ***

#include <RTSP.h>

#include <RTSP_Internals.h>

class PlantSend

{

public:

struct Update

{

double output;

uint _TimeTag;

ushort _SequenceNum;

uchar _ReceiverIndex;

uchar _SenderIndex;

bool Send(RTSP& rtsp, uchar rcv_index = _AllIndexes)

{ return _SendMessage(rtsp, this, 2, rcv_index); }

};

};

Note that the

## Generic Controller

The generic controller program is a Win32 graphical user interface that provides control and monitoring capability for any RTSP federation. The controller reads the MDF file that was used in building the federation and uses that information to transmit command messages and display responses received from the federates.

NOTE: If you have problems getting the controller to work with your federation, double check to be sure that the version of the MDF file the controller is reading matches the version used to build the federation exactly. Even the slightest difference in the MDF file may cause the controller not to work with the federation.

The controller application is named controller.exe. The syntax for running the controller is:

controller [file_name.mdf]

Here, the MDF file name is optional. If no MDF file name is given, the controller window will be empty upon startup. It is then necessary use the File menu Open option to select an MDF file. Once an MDF file is loaded, the names of the federates will appear in the window with the status "Not Connected" displayed next to each one.

Start all the federate applications running and then click the Ping button. You should get a status response from each federate in the form "Ping: x.xxx mS" which indicates the round trip ping time in milliseconds. If instead you get the message "Ping: No response" that means that the controller did not receive a response from that federate within 2 seconds. If this occurs, the federate may not be running or there may be a problem with the network.

After you can successfully ping all the federates, click Initialize to prepare for a simulation run. You should get the status indication "Initialize: Success" from each federate if all went well. The message "Initialize: No response" means that the controller did not receive a response in 2 seconds. The message "Initialize: Failed" means that the federate returned a status value to the controller indicating that its initialization attempt failed.

After you have successfully initialized all the federates, click Start to begin the simulation run. You should get the status indication "Start: Success" if all went well. The message "Start: No response" means that the controller did not receive a response in 2 seconds. The message "Start: Failed" means that the federate returned a status value to the controller indicating that its attempt to start execution failed.

After you have successfully started execution of the federation, you can click Halt to stop the simulation run at any time. You should get the status indication "Halt: Success" if all went well. The message "Halt: No response" means that the controller did not receive a response in 2 seconds. The message "Halt: Failed" means that the federate returned a status value to the controller indicating that its attempt to halt execution failed.

You can click Ping during simulation execution if you wish to verify continued connectivity of the federates. The processing required to respond to the Ping message should have a negligible effect on the performance of the federates.

# Example Federation

For this example, we will develop a federation consisting of three federates plus a system controller. The system to be modeled is a continuous plant with a continuous controller plus a command generator that drives the controller.

The plant is a "pure inertia" system with the transfer function . The controller is implemented with the transfer function . The command generator outputs a randomly selected command in the range 0-10 at ten second intervals.

We implement this federation with three federates plus a controller:

- The CommandGenerator federate creates and outputs the commands to the system.

- The Controller federate computes the error between the command value and the current system output and passes that error signal through the controller transfer function. The output of the controller transfer function drives the plant.

- The Plant federate accepts the controller output signal as its input and uses it to drive the plant transfer function.

- The generic controller application will control the operation of the entire federation.

Given this information we can develop an MDF file that describes the federates and the messages that pass between them:

// Distributed Simulation Message Definition

// Declare names for all federates:

federate CommandGenerator, Controller, Plant;

// Define messages. The federate named with each message is the sender.

message CommandGenerator.Update

{

double command;

};

message Controller.Update

{

double drive;

};

message Plant.Update

{

double output;

};

// Subscription list. Each federate can subscribe to (i.e., receive) any

// message including ones it produces.

// The Controller gets updates from the CommandGenerator and Plant

subscribe Controller : CommandGenerator.Update, Plant.Update;

// The Plant gets updates from the Controller

subscribe Plant : Controller.Update;

We then run the MDF translator program with this file as its input:

C:\Projects\DistSim>translator DistSim.mdf

Generating CommandGeneratorSend.h

Generating ControllerSend.h

Generating ControllerRcv.h

Generating PlantSend.h

Generating PlantRcv.h

Generating _MsgData.cpp

C:\Projects\DistSim>

The contents of CommandGeneratorSend.h are:

// CommandGeneratorSend class

// Generated Thu Aug 24 13:33:36 2000 from distsim.mdf

// *** This file was automatically generated. Do not edit it! ***

#include <RTSP.h>

#include <RTSP_Internals.h>

class CommandGeneratorSend

{

public:

struct Update

{

double command;

uint _TimeTag;

ushort _SequenceNum;

uchar _ReceiverIndex;

uchar _SenderIndex;

bool Send(RTSP& rtsp, uchar rcv_index = _AllIndexes)

{ return _SendMessage(rtsp, this, 0, rcv_index); }

};

};

As we can see, the "output" member of the Update message appears in the Update structure, along with several parameters used internally by RTSP. The RTSP parameter names all start with underscores. The MDF file rules prevent the user defining variable names that start with underscores, so there is no chance of a name collision.

The contents of ControllerSend.h are:

// ControllerSend class

// Generated Thu Aug 24 13:33:36 2000 from distsim.mdf

// *** This file was automatically generated. Do not edit it! ***

#include <RTSP.h>

#include <RTSP_Internals.h>

class ControllerSend

{

public:

struct Update

{

double drive;

uint _TimeTag;

ushort _SequenceNum;

uchar _ReceiverIndex;

uchar _SenderIndex;

bool Send(RTSP& rtsp, uchar rcv_index = _AllIndexes)

{ return _SendMessage(rtsp, this, 1, rcv_index); }

};

};

The contents of ControllerRcv.h are:

// ControllerRcv class

// Generated Thu Aug 24 13:33:36 2000 from distsim.mdf

// *** This file was automatically generated. Do not edit it! ***

#include <RTSP.h>

#include <RTSP_Internals.h>

class ControllerRcv

{

public:

struct CommandGeneratorUpdate

{

double command;

uint _TimeTag;

ushort _SequenceNum;

uchar _ReceiverIndex;

uchar _SenderIndex;

void SetCallback(RTSP& rtsp, void (*_f)())

{ _AddCallback(rtsp, _f, this, 0); }

};

struct PlantUpdate

{

double output;

uint _TimeTag;

ushort _SequenceNum;

uchar _ReceiverIndex;

uchar _SenderIndex;

void SetCallback(RTSP& rtsp, void (*_f)())

{ _AddCallback(rtsp, _f, this, 2); }

};

};

The contents of PlantSend.h are:

// PlantSend class

// Generated Thu Aug 24 13:33:36 2000 from distsim.mdf

// *** This file was automatically generated. Do not edit it! ***

#include <RTSP.h>

#include <RTSP_Internals.h>

class PlantSend

{

public:

struct Update

{

double output;

uint _TimeTag;

ushort _SequenceNum;

uchar _ReceiverIndex;

uchar _SenderIndex;

bool Send(RTSP& rtsp, uchar rcv_index = _AllIndexes)

{ return _SendMessage(rtsp, this, 2, rcv_index); }

};

};

The contents of PlantRcv.h are:

// PlantRcv class

// Generated Thu Aug 24 13:33:36 2000 from distsim.mdf

// *** This file was automatically generated. Do not edit it! ***

#include <RTSP.h>

#include <RTSP_Internals.h>

class PlantRcv

{

public:

struct ControllerUpdate

{

double drive;

uint _TimeTag;

ushort _SequenceNum;

uchar _ReceiverIndex;

uchar _SenderIndex;

void SetCallback(RTSP& rtsp, void (*_f)())

{ _AddCallback(rtsp, _f, this, 1); }

};

};

The contents of _MsgData.cpp are used internally by the RTSP software and are not generally of interest to simulation developers (except perhaps when troubleshooting), so we will not discuss that file here.

Given these generated files, we must now develop the source code for each federate.

The CommandGenerator federate is implemented in the file CommandGenerator.cpp:

#include <RTSP.h>

#include <stdio.h>

#include "CommandGeneratorSend.h"

RTSP* rtsp;

CommandGeneratorSend::Update command_update;

bool ping_callback()

{

printf("Ping callback\n");

return true;

}

bool init_callback()

{

printf("Initialize callback\n");

return true;

}

bool start_callback()

{

printf("Start callback\n");

return true;

}

bool end_run = false;

bool halt_callback()

{

printf("Halt callback\n");

end_run = true;

return true;

}

int main()

{

printf("CommandGenerator Federate\n");

printf("Jim Ledin    August, 2000\n\n");

// Determine the number of frames per second

int frames_per_sec = 100;

printf("\nUpdate rate: %d frames/sec\n\n", frames_per_sec);

rtsp = new RTSP("CommandGenerator");

rtsp->SetPingCallback(ping_callback);

rtsp->SetInitializeCallback(init_callback);

rtsp->SetStartCallback(start_callback);

rtsp->SetHaltCallback(halt_callback);

for (;;)

{

end_run = false;

printf("Waiting for Initialize\n");

rtsp->WaitForInit();

printf("Waiting for Start\n");

rtsp->WaitForStart();

printf("Running\n");

double step_time = 1.0 / frames_per_sec;

rtsp->StartTimer();

uint frame = 0;

while (!end_run)

{

// Time since start of run

double Time = frame * step_time;

// Output a new random command every 10 seconds

if (frame % (10*frames_per_sec) == 0)

{

double command = 10.0 * double(rand()) / RAND_MAX;

command_update.command = command;

command_update.Send(*rtsp);

printf("Time: %.1lf; New command: %.3lf\n", Time, command);

fflush(stdout);

}

uint end_of_frame = uint(1e6 * ++frame * step_time);

uint cur_time = rtsp->ReadTimer();

while (cur_time < end_of_frame)

{

rtsp->PollRcv(true, end_of_frame - cur_time);

cur_time = rtsp->ReadTimer();

}

}

printf("End of run\n");

}

return 0;

}

The Controller federate is implemented in Controller.cpp. Note that we are using the DSSL library to implement the differential equations of the controller.

#include <dssl.h>

#include <RTSP.h>

#include <stdio.h>

#include "ControllerSend.h"

#include "ControllerRcv.h"

RTSP* rtsp;

ControllerSend::Update drive_update;

ControllerRcv::CommandGeneratorUpdate command_update;

ControllerRcv::PlantUpdate plant_update;

bool ping_callback()

{

printf("Ping callback\n");

return true;

}

bool init_callback()

{

printf("Initialize callback\n");

return true;

}

bool start_callback()

{

printf("Start callback\n");

return true;

}

bool end_run = false;

bool halt_callback()

{

printf("Halt callback\n");

end_run = true;

return true;

}

void command_callback()

{

printf("New command from CommandGenerator: %.2f\n",

command_update.command);

}

void plant_callback()

{

}

int main()

{

printf("Controller Federate\n");

printf("Jim Ledin    August, 2000\n\n");

// Determine the number of frames per second

int frames_per_sec = 100;

printf("\nUpdate rate: %d frames/sec\n\n", frames_per_sec);

rtsp = new RTSP("Controller");

rtsp->SetPingCallback(ping_callback);

rtsp->SetInitializeCallback(init_callback);

rtsp->SetStartCallback(start_callback);

rtsp->SetHaltCallback(halt_callback);

command_update.SetCallback(*rtsp, command_callback);

plant_update.SetCallback(*rtsp, plant_callback);

// Declare a list of state variables and put the state variables in it

StateList state_list;

State<> state(&state_list, StateBase::RK4);

double step_time = 1.0 / frames_per_sec;

for (;;)

{

end_run = false;

printf("Waiting for Initialize\n");

rtsp->WaitForInit();

// State initial value

state.ic = 0;

// Set the initial value of the states

state_list.Initialize(step_time);

command_update.command = 0;

printf("Waiting for Start\n");

rtsp->WaitForStart();

printf("Running\n");

rtsp->StartTimer();

uint frame = 0;

while (!end_run)

{

// Time since start of run

double Time = frame * step_time;

// Compensator transfer function

double input = command_update.command - plant_update.output;

state.der = -40*state + input;

double drive = -19000*state + 500*input;

drive_update.drive = drive;

drive_update.Send(*rtsp);

if (frame % frames_per_sec == 0)

printf("Time: %.2lf; Controller output: %.3lf\n",

Time, drive);

state_list.Integrate();

uint end_of_frame = uint(1e6 * ++frame * step_time);

uint cur_time = rtsp->ReadTimer();

while (cur_time < end_of_frame)

{

rtsp->PollRcv(true, end_of_frame - cur_time);

cur_time = rtsp->ReadTimer();

}

}

printf("End of run\n");

}

return 0;

}

Finally, we implement the plant model, again using the DSSL library to implement the differential equations of the plant in Plant.cpp:

#include <dssl.h>

#include <RTSP.h>

#include <stdio.h>

#include "PlantSend.h"

#include "PlantRcv.h"

RTSP* rtsp;

PlantSend::Update plant_update;

PlantRcv::ControllerUpdate drive_update;

bool ping_callback()

{

printf("Ping callback\n");

return true;

}

bool init_callback()

{

printf("Initialize callback\n");

return true;

}

bool start_callback()

{

printf("Start callback\n");

return true;

}

bool end_run = false;

bool halt_callback()

{

printf("Halt callback\n");

end_run = true;

return true;

}

void drive_callback()

{

}

int main()

{

printf("Plant Federate\n");

printf("Jim Ledin    August, 2000\n\n");

// Determine the number of frames per second

int frames_per_sec = 100;

printf("\nUpdate rate: %d frames/sec\n\n", frames_per_sec);

rtsp = new RTSP("Plant");

rtsp->SetPingCallback(ping_callback);

rtsp->SetInitializeCallback(init_callback);

rtsp->SetStartCallback(start_callback);

rtsp->SetHaltCallback(halt_callback);

drive_update.SetCallback(*rtsp, drive_callback);

// Declare a list of state variables and put the state variables in it

StateList state_list;

State<> pos(&state_list);

State<> vel(&state_list);

double step_time = 1.0 / frames_per_sec;

for (;;)

{

end_run = false;

printf("Waiting for Initialize\n");

rtsp->WaitForInit();

// State initial values

pos.ic = 0;

vel.ic = 0;

// Set the initial value of the states

state_list.Initialize(step_time);

drive_update.drive = 0;

printf("Waiting for Start\n");

rtsp->WaitForStart();

printf("Running\n");

rtsp->StartTimer();

uint frame = 0;

while (!end_run)

{

// Time since start of run

double Time = frame * step_time;

// Use the most recent value of the controller drive signal

vel.der = drive_update.drive;

pos.der = vel;

plant_update.output = pos;

plant_update.Send(*rtsp);

state_list.Integrate();

if (frame % frames_per_sec == 0)

printf("Time: %.2lf; Plant output: %.3lf\n", Time, double(pos));

uint end_of_frame = uint(1e6 * ++frame * step_time);

uint cur_time = rtsp->ReadTimer();

while (cur_time < end_of_frame)

{

rtsp->PollRcv(true, end_of_frame - cur_time);

cur_time = rtsp->ReadTimer();

}

}

printf("End of run\n");

}

return 0;

}

We compile the three programs above and link each of them with the code in the _MsgData.cpp and the provided RTSP.cpp source files. Since the controller and plant use the DSSL library, these programs must also link with the code provided in the StateList.cpp file in the DSSL distribution.

Having built these three applications, we can now execute them as a federation along with a federation controller application. Initially, we can run everything on a single computer. On a Windows NT system we can run the three program executables plus the generic controller application from a batch file (named run.bat) as follows:

start controller DistSim.mdf

start CommandGenerator

start Controller

start Plant

The run.bat file listed above assumes that all the executable programs and the DistSim.mdf file are located in the current directory.

Once all four programs are running, the three federate names should be listed in the generic controller window. Click the Ping button to verify that all federates respond. Then click Initialize and verify that all federates report successful initialization. Click Start to start real time execution. The plant and controller transmit Update messages at a 100 Hz. rate and display on-screen information at a 1 Hz. rate. The CommandGenerator transmits a new command every 10 seconds and displays each new command as it is transmitted.

Observe that the plant output converges to the commanded value within about three seconds after each new command is issued by the CommandGenerator. The Controller output decays to zero within about three seconds of each new command as well.

To distribute the simulation across a network, simply terminate one of the federate applications and start the same executable on a different computer system on the network. As soon as the federate is running on the other computer you can click Ping to verify connectivity and perform a simulation run as discussed above.

You should be able to compile the source code for these federates with minimal (or no) changes for execution on a Unix system. Execute one or more of the federates on a Unix system and repeat the steps above to run the simulation.

This simple federation has not exercised many of the more sophisticated capabilities possible with RTSP. Some of the areas you may wish to explore further are:

- Enable logging of message traffic to disk during simulation execution. Develop and application to read the log file and analyze it after the simulation run completes.

- Synchronize the time on multiple computers using GPS or some other precision timing reference.

- Extrapolate continuous variables to estimate their value at the current time. This requires that all federates execute on a single computer or that some type of distributed precision time reference be in use.

- Real time collection and post-run display of information about sequences of lost messages.

- Develop a real time viewer application to display the state of the entire federation using the information passed in the messages between the federates. This will require that a new federate be added to the MDF file that subscribes to all messages from all federates. This viewer federate receives the messages and displays the information they contain in real time.

# Miscellaneous Notes

## Building a program that dynamically loads MDF data

In the discussion above, we have assumed that the MDF data placed in the _MsgData.cpp source file was compiled into each federate. This is the simplest approach and it produces the smallest and fastest applications for RTSP. However, the translator program and generic controller application do not use this approach and it is not mandatory that federates use it either. It is possible to load the MDF information directly from the MDF text file.

For insight into how to dynamically load MDF data, examine the translator program source code provided with the RTSP distribution.

The relevant parts of the translator.cpp source file are shown here:

#include <Parser.h>

int main(int argc, char *argv[])

{

if (argc < 2)

{

fprintf(stderr, "usage: %s filename.mdf\n", argv[0]);

exit(1);

}

ParseMDFFile(argv[1]);

GenerateCppCode();

return 0;

}

As you can see, loading the MDF data simply requires calling the ParseMDFFile function defined in Parser.h. You must also compile and link in the source files contained in the Lib directory of the RTSP distribution. Finally, the preprocessor symbol RTSP_DATA_MODIFIABLE must be defined when compiling all the source files for this application.

Unlike using static data generated by the MDF translator, programs that dynamically load MDF files will perform dynamic memory allocation and the code size will be significantly larger since the MDF parser is included.

## FD_SETSIZE

This preprocessor symbol is set to the value of 128 in RTSP.h and it indicates the maximum number of TCP/IP sockets that can be in an FD_SET. For the purposes of RTSP, this is the maximum number of different receive messages that a federate can have. If more than 128 are needed, change the value in rtsp.h to a larger value and recompile the federate’s source code. The maximum possible value for FD_SETSIZE is system-dependent.

Information about the High Level Architecture is available at http://www.dmso.mil/hla.